/*

Mini DSO with STC MCU

MCU Type: STC8A8K64S4A12 @27MHz

Operation method:

Main Interface - Parameter Mode:

Single Click Encoder: Run/Stop sampling

Double Click Encoder: Enter Wave Scroll Mode

Long Press Encoder: Enter Settings Interface

Rotate Encoder: Adjust parameters

Rotate Encoder While Pressing: Switch between options

Main Interface - Wave Scroll Mode:

Single Click Encoder: Run/Stop sampling

Double Click Encoder: Enter Parameter Mode

Long Press Encoder: Enter Settings Interface

Rotate Encoder: Scroll waveform horizontally (only available when sampling stopped)

Rotate Encoder While Pressing: Scroll waveform vertically (only available when sampling stopped)

Settings Interface:

Single Click Encoder: N/A

Double Click Encoder: N/A

Long Press Encoder: Return to Main Interface

Rotate Encoder: Adjust parameters

Rotate Encoder While Pressing: Switch between options

Parameters in Main Interface:

Seconds Per Division: "500ms", "200ms", "100ms", "50ms", "20ms", "10ms","5ms", "2ms", "1ms", "500us", "200us", "100us"(100us only available in Auto Trigger Mode)

Voltage Range: Maximum voltage is 27V. Rotate Encoder clockwise continuous to enter auto range. Rotate Encoder anticlockwise to enter manual range.

Status in Main Interface:

'Run': Sampling Running.

'Stop': Sampling Stopped.

'Fail': The Trigger Level beyond the waveform in Auto Trigger Mode.

'Auto': Auto Voltage Range.

Parameters in Settings Interface:

PMode(Plot Mode): Show waveform in Vector or Dots.

LSB(Sampling Coefficient): 100 times of voltage dividing coefficient. e.g. the resistor for voltage dividing is 11k and 2.2k, calculate the voltage dividing coefficient (11+2.2)/2=6.6. Get the LSB=6.6x100=660.

BRT(Brightness): Adjust OLED Brightness

Saving Settings: When exit settings interface, all parameters in settings and main interface will be saved in EEPROM.

Indicator:

Generally, the indicator on means the sampling is running. The more important use is in Single and Normal Trigger Mode, before get into the trigger stage, pre-sampling is required. The indicator will not on during pre-sampling stage. We should not input signal until the indicator comes on. The longer time scale selected, the longer waiting time of pre-sampling.

*/

#include <reg51.h>

#include "global.h"

#include "delay.h"

#include "adc_stc8.h"

#include "config_stc.h"

#include "settings.h"

#include "chart.h"

#include "ssd1306.h"

// #include "uart.h"

// #include "stdio.h"

#define VBAT_UPDATE_FREQ 40 //The update frequency of the battery voltage information VBAT_UPDATE_FREQ*25ms

#define VBAT_LSB 660        //Battery voltage sampling voltage divider coefficient*100

#define INT_0 0             //External Interrupt 0£¬Rotate Encoder

#define INT_1 2             //External Interrupt 1£¬Press Encoder

#define TIMER_0 1           //Timer interrupt 0

void init()

{

InSettings = 0;         //Initialize the boot interface

OptionInSettings = 0;   //Initialize the option number of the setting interface

OptionInChart = 0;      //Initialize the option number of the main interface

WaveScroll = 0;         //Clear 'waveform scrolling'

OptionChanged = 0;      //Clear 'setting modification'

ADCRunning = 1;         //Set sampling

ADCInterrupt = 0;       //Clear 'ADC interrupt'

ADCComplete = 0;        //Clear 'ADC complete'

WaveUpdate = 0;         //Clear 'wave update'

ClearWave = 0;          //Clear 'clear wave'

ClearDisplay = 1;       //Set 'clear display'

EC11PressAndRotate = 0;  //Clear encoder press and rotate

UpdateVbat = 0;         //Clear sampling battery voltage

WaveFreq = 0;           //Clear wave frequence

TriPosOffset = 0;       //Initialize trigger point offset

TriFail = 0;            //Clear the sign of triggering failed

VMax = 0;               //Clear the maximum of the Voltage

VMin = 0;               //Clear the minimum of the Voltage

WaveLengthSumNum = 0;   //Clear the times of waveform length summation

WaveLengthSum = 0;      //Clear waveform length summation

/* Disable external interrupt 0(Encoder rotation) in waveform scroll mode when sampling*/

if (ADCRunning && WaveScroll)

EX0 = 0;

/* Get internal reference voltage */

BGV = GetBGV();

}

void runWhenInSettrings()

{

P_Ready = 0;

ClearDisplay = 1;

while (InSettings)

{

/* Update voltage infor of battery */

if (UpdateVbat)

{

UpdateVbat = 0;

VBat = GetVoltage(ADC_BAT, VBAT_LSB);

}

/* Update OLED Brightness */

if (OLED_BrightnessChanged)

{

OLED_BrightnessChanged = 0;

OLED_Write_Command(0x81);

OLED_Write_Command(OLED_Brightness * 10);

}

/* Update parameters */

if (DisplayUpdate)

{

DisplayUpdate = 0;

PlotSettings();

OLED_Display();

}

}

/* Save Settings */

EA = 0;

if (OptionChanged)

{

OptionChanged = 0;

ClearDisplay = 0;

/* Save settings and display saving status*/

PlotSaveStatus(Save_Options());

OLED_Display();

Delay1000ms();

}

/* Redraw display after exit settings */

ClearDisplay = 1;

PlotChart();

OLED_Display();

IE0 = 0;

IE1 = 0;

EA = 1;

}

/*

ADCRuning=1

|	WaveUpdate=1

|	|	ClearWave=1

|	|	ClearWave=0

|	WaveUpdate=0

ADCRuning=0

DisplayUpdate=1

WaveUpdate=1

|	ClearWave=1

|	ClearWave=0

WaveUpdate=0

*/

void runWhenADCInterrupt()

{

ADCInterrupt = 0;

P_Ready = 0;

/* ADC Sampling Running */

if (ADCRunning)

{

EX0 = 0;

if (WaveUpdate)

{

WaveUpdate = 0;

ClearDisplay = 1;

/* Clear waveform in buffer*/

if (ClearWave)

{

ClearWave = 0;

memset(ADCbuf, 0x00, SAMPLE_NUM * 2);

WaveFreq = 0;

TriFail = 0;

VMax = 0;

VMin = 0;

PlotChart();

}

/* Display waveform in buffer*/

else

{

AnalyseData();

PlotChart();

PlotWave();

}

}

else

{

ClearDisplay = 0;

PlotChart();

}

OLED_Display();

/* Not to enable external interrupt 0(Encoder Rotation) when both of ADCRunning and WaveScroll set. */

if (!WaveScroll)

{

IE0 = 0;

EX0 = 1;

}

}

/* ADC Sampling Stopped */

else

{

while (!ADCRunning && !InSettings)

{

if (DisplayUpdate)

{

EX0 = 0;

DisplayUpdate = 0;

if (WaveUpdate)

{

WaveUpdate = 0;

/* Analyse completed sampling data in buffer */

AnalyseData();

ClearDisplay = 1;

/* Clear display and draw waveform */

if (ScaleH == ScaleH_tmp)

{

/* Since the sampling points limitation, scaling waveform along horizontal is not support.

Show waveform only when time scale is same as that when sampling completed */

PlotChart();

PlotWave();

}

/* Clear display and no waveform */

else

{

WaveFreq = 0; //Clear WaveFreq. When the interface is drawn, the screen will display xxHZ

PlotChart();

}

}

/* Update parameters on display only, keep waveform */

else

{

ClearDisplay = 0;

PlotChart();

}

OLED_Display();

IE0 = 0;

EX0 = 1;

}

}

}

}

void runWhenADCComplete()

{

/*  ADC Sampling Complete - Single Trigger Mode */

if (TriMode == 2)

{

EX0 = 0;

ADCInterrupt = 1;

DisplayUpdate = 0;

WaveUpdate = 0;

ADCRunning = 0;    //Clear ACDRunning and stop sampling

AnalyseData();

ClearDisplay = 1;

PlotChart();

PlotWave();

OLED_Display();

IE0 = 0;

EX0 = 1;

}

/*  ADC Sampling Complete - Auto or Normal Trigger Mode */

else

{

//EX0 = 0;

ClearDisplay = 1;   //Set to clear display

AnalyseData();    //analyse the data of sampling

PlotChart();       //Draw the main interface

PlotWave();       //Draw waveform

OLED_Display();

//IE0 = 0;

//EX0 = 1;

}

}

/* Program Entry */

void main()

{

/* Set ADC port P0.4, P0.6 as high-impedance */

P0M1 |= 0x50;

P0M0 &= ~0x50;

/* Set indicator port P1.1 as push-pull output */

P0M1 &= ~0x02;

P0M0 |= 0x02;

/* Set Encoder port P3.2, P3.3, P3.4 as quasi bidirectional */

P3M1 &= ~0x1C;

P3M0 &= ~0x1C;

/* Set OLED port P2.3, P2.4, P2.5, P2.6 as quasi bidirectional */

P2M1 &= ~0xF8;

P2M0 &= ~0xF8;

/* Interrupt for rotating of Encoder */

IT0 = 0;  //Rising edge triggered for external interrupt 0

PX0 = 1;  //High priority for external interrupt 0. Used to judge when pressing and rotating at the same time

EX0 = 1;  //Enable external interrupt 0

/* Interrupt for clicking of Encoder */

IT1 = 1;    //Falling edge triggered for external interrupt 1

PX1 = 0;   //Low priority for external interrupt 1.

EX1 = 1;   //Enable external interrupt 1

/* Timer 0, for updating voltage of battery */

AUXR &= 0x7F;  //12T mode for timer

TMOD &= 0xF0;  //Set mode for timer

TL0 = 0x46;    //Set the timing initial value //27MHz£¬25ms

TH0 = 0x24;   //Set the timing initial value

TF0 = 0;      //Clear TF0

ET0 = 1;      //Enable timer 0 interrupt

/* Enable global interrupt */

EA = 1;

//UartInit();

Read_Options();

init();

OLED_Init();

OLED_SetFontWidth(6);

PlotChart();

OLED_Display();

while (1)

{

P_Ready = 0;

/* Sampling*/

GetWaveData();

/* Enter Settings Interface */

if (InSettings)

{

runWhenInSettrings();

}

/* ADC Sampling Interrupt */

else if (ADCInterrupt)

{

runWhenADCInterrupt();

}

/* ADC Sampling Complete */

else

{

runWhenADCComplete();

}

}

}

/* Analyse Rotation of Encoder */

void Scan_EC11(void)

{

/* Delay to remove jitter */

Delay500us();

/* Clockwise */

if (EC11_A != EC11_B)

{

Change_Val(1);

}

/* Anticlockwise */

else if (EC11_A == EC11_B)

{

Change_Val(0);

}

}

/* Interrput for Encoder Rotated */

void INT0_interrupt(void) interrupt INT_0

{

Delay500us();

/* Whether the Encoder is pressed */

if (!EC11_KEY)

EC11PressAndRotate = 1;

else

EC11PressAndRotate = 0;

Scan_EC11();

ADCInterrupt = 1;

DisplayUpdate = 1;

IE1 = 0; //Clear external interrupt 1

IE0 = 0; //Clear external interrupt 0

}

/* Interrput for Encoder Pressed */

void INT1_interrupt(void) interrupt INT_1

{

Delay50ms();

if (!EC11_KEY)

{

EC11PressAndRotate = 0;

/*  Long presse Encoder - Switch main interface and settings interface */

if (Delay800ms_Long_Press())

{

InSettings = ~InSettings;

/*  Enter Settings  */

if (InSettings)

{

DisplayUpdate = 1;

UpdateVbat = 1;

TF0 = 0;  //Clear timer overflow

TR0 = 1;  //Timer 0 starts counting. Start battery voltage information update timer

IE0 = 0;  //Clear the external interrupt 0

EX0 = 1;  //Open the external interrupt 0 (rotation of encoder)

}

/*  Retrurn to main interface   */

else

{

TR0 = 0;  //Clear timer overflow

TF0 = 0;  //When timer 0 stops, stop the battery voltage information update timer

WaveFreq = 0;

TriFail = 0;

VMax = 0;

VMin = 0;

DisplayUpdate = 1;

WaveUpdate = 1;

ClearWave = 0;

}

ADCInterrupt = 1;

}

/*  Rotate Encoder while pressing  */

else if (EC11PressAndRotate)

{

/* Operations Performed by Interrupt of Encoder Rotation */

}

/* Double click Encoder - Switch between Waveform Scroll Mode and Parameter Mode in Main Interface */

else if (Delay300ms_Double_Click())

{

/* Main Interface*/

if (!InSettings)

{

WaveScroll = ~WaveScroll;

OptionChanged = 1;

ADCInterrupt = 1;

DisplayUpdate = 1;

ClearWave = 0;

}

}

/*  Single click Encoder - Switch Run/Stop in main interface    */

else if (!InSettings)

{

EX0 = 0;

ADCRunning = ~ADCRunning;

if (ADCRunning)

{

WaveUpdate = 1;

ClearWave = 1;

}

else

{

DisplayUpdate = 1;

WaveUpdate = 1;

}

ADCInterrupt = 1;

IE0 = 0;

IE1 = 0;

}

}

IE1 = 0;

}

/* Timer for updating Vbat */

void TIMER0_interrupt(void) interrupt TIMER_0

{

static uint8 n;

if (++n >= VBAT_UPDATE_FREQ)

{

n = 0;

UpdateVbat = 1;

DisplayUpdate = 1;

}

}